基础：

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BOARD)

GPIO.setup(8, GPIO.OUT) #R GPIO14

GPIO.setup(10, GPIO.OUT) #G GPIO15

GPIO.setup(12, GPIO.OUT) #B GPIO18

GPIO.setup(3, GPIO.IN) #S GPIO2

try:

while True:

if(GPIO.input(3)==1):

GPIO.output(8,1)

GPIO.output(10,0)

else:

GPIO.output(8,0)

GPIO.output(10,1)

except KeyboardInterrupt:

print("用户终止程序")

finally:

GPIO.cleanup()

print("程序退出，GPIO已清理")

扩展：

import RPi.GPIO as GPIO

import time

led_state = 0

GPIO.setmode(GPIO.BOARD)

GPIO.setup(8, GPIO.OUT) #R GPIO14

GPIO.setup(10, GPIO.OUT) #G GPIO15

GPIO.setup(12, GPIO.OUT) #B GPIO18

GPIO.setup(3, GPIO.IN) #S GPIO2

try:

while True:

if (GPIO.input(3)==0):

time.sleep(0.01)

if GPIO.input(3)==0:

if led_state==0:

led_state=1

print("Red")

elif led_state==1:

led_state=2

print("Red Shining")

elif led_state==2:

led_state=3

print("Green")

elif led_state==3:

led_state=4

print("Green Shining")

elif led_state==4:

led_state=5

print("Blue")

elif led_state==5:

led_state=6

print("Blue Shining")

elif led_state==6:

led_state=0

print("close")

time.sleep(1)

if led_state==0:

GPIO.output(8, 0)

GPIO.output(10, 0)

GPIO.output(12, 0)

elif led_state==1:

GPIO.output(8, 1)

GPIO.output(10, 0)

GPIO.output(12, 0)

elif led_state==2:

GPIO.output(8, 1)

GPIO.output(10, 0)

GPIO.output(12, 0)

time.sleep(0.2)

GPIO.output(8, 0)

time.sleep(0.2)

GPIO.output(8, 1)

elif led_state==3:

GPIO.output(8, 0)

GPIO.output(10, 1)

GPIO.output(12, 0)

elif led_state==4:

GPIO.output(8, 0)

GPIO.output(10, 1)

GPIO.output(12, 0)

time.sleep(0.2)

GPIO.output(10, 0)

time.sleep(0.2)

GPIO.output(10, 1)

elif led_state==5:

GPIO.output(8, 0)

GPIO.output(10, 0)

GPIO.output(12, 1)

elif led_state==6:

GPIO.output(8, 0)

GPIO.output(10, 0)

GPIO.output(12, 1)

time.sleep(0.2)

GPIO.output(12, 0)

time.sleep(0.2)

GPIO.output(12, 1)

except KeyboardInterrupt:

print("用户终止程序")

finally:

GPIO.cleanup()

print("程序退出，GPIO已清理")