import smbus

import time

import RPi.GPIO as GPIO

#设备地址

DEVICE_ADDR = 0x48

#0对Y，1对X

CH_VRX = 1

CH_VRY = 0

CENTER   = 128

DEADZONE = 10

GPIO.setmode(GPIO.BCM)

PIN_RED   = 18

PIN_GREEN = 19

GPIO.setup(PIN_RED, GPIO.OUT)

GPIO.setup(PIN_GREEN, GPIO.OUT)

pwm_red = GPIO.PWM(PIN_RED, 2000)

pwm_green = GPIO.PWM(PIN_GREEN, 2000)

bus = smbus.SMBus(1)

def read_adc(channel: int) -> int:

ctrl_byte = channel

bus.write_byte(DEVICE_ADDR, ctrl_byte)

bus.read_byte(DEVICE_ADDR)

return bus.read_byte(DEVICE_ADDR)

def brightness(raw: int) -> float:

return max(0.0, min(100.0, raw / 255.0 * 100.0))

mode = "RED"

red_bri = 50.0

green_bri = 0.0

pwm_red.start(red_bri)

pwm_green.start(green_bri)

print(f"State=Neutral(RED), X=?, Y=?, Red={red_bri:.1f}%, Green={green_bri:.1f}%")

try:

while True:

vrx = read_adc(CH_VRX)

vry = read_adc(CH_VRY)

dx = vrx - CENTER

dy = vry - CENTER

state = "Neutral"

if abs(dx) > abs(dy) and abs(dx) > DEADZONE:

mode = "GREEN"

green_bri = brightness(255 - vrx)

red_bri = 0.0

if dx < 0:

state = "Left"

else:

state = "Right"

elif abs(dy) > DEADZONE:

mode = "RED"

red_bri = brightness(255 - vry)

green_bri = 0.0

if dy < 0:

state = "Up"

else:

state = "Down"

else:

if mode == "RED":

state = "center(RED)"

red_bri = 50.0

green_bri = 0.0

else:

state = "center(GREEN)"

red_bri = 0.0

green_bri = 50.0

pwm_red.ChangeDutyCycle(red_bri)

pwm_green.ChangeDutyCycle(green_bri)

print(

f"place={state}, "

f"X={vrx:3d}, Y={vry:3d}, "

f"Red={red_bri:6.1f}%, Green={green_bri:6.1f}%"

)

time.sleep(0.1)

except KeyboardInterrupt:

print("Exit by Ctrl+C")

finally:

pwm_red.stop()

pwm_green.stop()

GPIO.cleanup()