Key_Onetape

Main.c

/*********************************

芯片：STM32F103VET6

实现功能：LED灯按键闪烁（红灯）

引脚：PB5(红),key1按键

1. include stm32f10.h、led.h

2. 准备 delay 函数

3. 准备 key_scan 函数

4. 打开对应端口

5. 设置初始值

6. 进入循环 while(1)

7. 检测到按键，翻转灯的状态

*************************************/

#include "stm32f10x.h"

#include "led.h"

#include "delay.h"

#include "key.h"

#define KEY_ON  1

#define KEY_OFF 0

int main(void)

{

/* LED 端口初始化 */

LED_GPIO_R_Config();

/*初始化按键*/

Key_GPIO_Config();

LED_R_OFF();

/* 轮询按键状态，若按键按下则反转LED */

while (1)

{

if ( Key_Scan(KEY1_GPIO_PORT,GPIO_Pin_0) == KEY_ON  )

{

LED_R_TOGGLE();

}

if ( Key_Scan(KEY2_GPIO_PORT,GPIO_Pin_13) == KEY_ON)//作为重置按钮，回到红灯状态

{

LED_R_ON();

}

}

}

Key.c

#include "key.h"

#include "stm32f10x.h"

#include "delay.h"

#include "led.h"

#define KEY_ON  1

#define KEY_OFF 0

void Key_GPIO_Config(void)  //按键初始化

{

GPIO_InitTypeDef GPIO_InitStructure;

/*开启按键端口的时钟*/

RCC_APB2PeriphClockCmd(KEY1_GPIO_CLK|KEY2_GPIO_CLK,ENABLE);

//选择按键的引脚

GPIO_InitStructure.GPIO_Pin = KEY1_GPIO_PIN;

// 设置按键的引脚为浮空输入

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING;

//使用结构体初始化按键

GPIO_Init(KEY1_GPIO_PORT, &GPIO_InitStructure);

//选择按键的引脚

GPIO_InitStructure.GPIO_Pin = KEY2_GPIO_PIN;

//设置按键的引脚为浮空输入

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING;

//使用结构体初始化按键

GPIO_Init(KEY2_GPIO_PORT, &GPIO_InitStructure);

}

uint8_t Key_Scan(GPIO_TypeDef* GPIOx,uint16_t GPIO_Pin)

{

/*检测是否有按键按下 */

if (GPIO_ReadInputDataBit(GPIOx,GPIO_Pin) == KEY_ON ) {

/*等待按键释放 */

while (GPIO_ReadInputDataBit(GPIOx,GPIO_Pin) == KEY_ON);

return  KEY_ON;

} else

return KEY_OFF;

}

Key.h

#ifndef __KEY_H

#define __KEY_H

#include "stm32f10x.h"

#include "stm32f10x.h"

#define    KEY1_GPIO_CLK     RCC_APB2Periph_GPIOA

#define    KEY1_GPIO_PORT    GPIOA

#define    KEY1_GPIO_PIN     GPIO_Pin_0  //PA0

#define    KEY2_GPIO_CLK     RCC_APB2Periph_GPIOC

#define    KEY2_GPIO_PORT    GPIOC

#define    KEY2_GPIO_PIN     GPIO_Pin_13  //PC13

void Key_GPIO_Config(void);

uint8_t Key_Scan(GPIO_TypeDef *GPIOx,uint16_t GPIO_Pin);

#endif

Led.c

#include "led.h"

void LED_GPIO_R_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin5;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_GPIO_G_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin0;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_GPIO_B_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin1;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_R_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin5); //LED on

}

void LED_R_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin5); //LED off

}

void LED_G_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin0); //LED on

}

void LED_G_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin0); //LED off

}

void LED_B_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin1); //LED on

}

void LED_B_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin1); //LED off

}

void LED_R_TOGGLE(void)

{

if(GPIO_ReadOutputDataBit(LED_GPIO_PORT, LED_GPIO_Pin5) == Bit_RESET)

{

LED_R_OFF();

}

else

{

LED_R_ON();

}

}

Led.h

#ifndef __LED_H

#define __LED_H

#include "stm32f10x.h"

#define LED_GPIO_CLK   RCC_APB2Periph_GPIOB

#define LED_GPIO_Pin5   GPIO_Pin_5 //R

#define LED_GPIO_Pin0   GPIO_Pin_0 //G

#define LED_GPIO_Pin1   GPIO_Pin_1 //B

#define LED_GPIO_PORT   GPIOB

void LED_GPIO_R_Config(void);

void LED_GPIO_G_Config(void);

void LED_GPIO_B_Config(void);

void LED_R_ON(void);

void LED_R_OFF(void);

void LED_G_ON(void);

void LED_G_OFF(void);

void LED_B_ON(void);

void LED_B_OFF(void);

void LED_R_TOGGLE(void);

#endif

Key_Transfer

Main.c

/*********************************

芯片：STM32F103VET6

实现功能：LED灯闪烁

引脚：PB5(红),PB0(绿),PB1(蓝) (RGB) 000-111(八种状态）

*************************************/

#include "stm32f10x.h"

#include "led.h"

#include "delay.h"

#include "key.h"

#define KEY_ON  1

#define KEY_OFF 0

int main(void)

{

int count = 0;

/* LED 端口初始化 */

LED_GPIO_R_Config();

LED_GPIO_G_Config();

LED_GPIO_B_Config();

/*初始化按键*/

Key_GPIO_Config();

//题目要求控制红灯的状态,初始化不亮灯

LED_R_OFF();

LED_G_OFF();

LED_B_OFF();

/*如果要在八个状态间变换

从红灯状态为初始状态100

100-101：

*/

/* 轮询按键状态，若按键按下则反转LED */

while (1)

{

if ( Key_Scan(KEY1_GPIO_PORT,GPIO_Pin_0) == KEY_ON  )

{

count = count+1;

if(count>=8)

{

count = count-8;

}

}

if ( Key_Scan(KEY2_GPIO_PORT,GPIO_Pin_13) == KEY_ON)//作为重置按钮，回到红灯状态

{

count =0;

}

if(count==0)//100

{

LED_R_ON();

LED_G_OFF();

LED_B_OFF();

}

if(count==1)//101

{

LED_R_ON();

LED_G_OFF();

LED_B_ON();

}

if(count==2)//110

{

LED_R_ON();

LED_G_ON();

LED_B_OFF();

}

if(count==3)//111

{

LED_R_ON();

LED_G_ON();

LED_B_ON();

}

if(count==4)//000

{

LED_R_OFF();

LED_G_OFF();

LED_B_OFF();

}

if(count==5)//001

{

LED_R_OFF();

LED_G_OFF();

LED_B_ON();

}

if(count==6)//010

{

LED_R_OFF();

LED_G_ON();

LED_B_OFF();

}

if(count==7)//011

{

LED_R_OFF();

LED_G_ON();

LED_B_ON();

}

}

}

Key.c

#include "key.h"

#include "stm32f10x.h"

#include "delay.h"

#include "led.h"

#define KEY_ON  1

#define KEY_OFF 0

void Key_GPIO_Config(void)  //按键初始化

{

GPIO_InitTypeDef GPIO_InitStructure;

/*开启按键端口的时钟*/

RCC_APB2PeriphClockCmd(KEY1_GPIO_CLK|KEY2_GPIO_CLK,ENABLE);//“|”开PA，PC端口时钟

//选择按键的引脚

GPIO_InitStructure.GPIO_Pin = KEY1_GPIO_PIN;

// 设置按键的引脚为浮空输入

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING;

//使用结构体初始化按键

GPIO_Init(KEY1_GPIO_PORT, &GPIO_InitStructure);

//选择按键的引脚

GPIO_InitStructure.GPIO_Pin = KEY2_GPIO_PIN;

//设置按键的引脚为浮空输入

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN_FLOATING;

//使用结构体初始化按键

GPIO_Init(KEY2_GPIO_PORT, &GPIO_InitStructure);

}

uint8_t Key_Scan(GPIO_TypeDef* GPIOx,uint16_t GPIO_Pin)

{

/*检测是否有按键按下 */

if (GPIO_ReadInputDataBit(GPIOx,GPIO_Pin) == KEY_ON ) {

/*等待按键释放 */

while (GPIO_ReadInputDataBit(GPIOx,GPIO_Pin) == KEY_ON);

return  KEY_ON;

} else

return KEY_OFF;

}

Key.h

#ifndef __KEY_H

#define __KEY_H

#include "stm32f10x.h"

#include "stm32f10x.h"

#define    KEY1_GPIO_CLK     RCC_APB2Periph_GPIOA

#define    KEY1_GPIO_PORT    GPIOA

#define    KEY1_GPIO_PIN     GPIO_Pin_0  //PA0

#define    KEY2_GPIO_CLK     RCC_APB2Periph_GPIOC

#define    KEY2_GPIO_PORT    GPIOC

#define    KEY2_GPIO_PIN     GPIO_Pin_13  //PC13

void Key_GPIO_Config(void);

uint8_t Key_Scan(GPIO_TypeDef *GPIOx,uint16_t GPIO_Pin);

#endif

Led.c

#include "led.h"

void LED_GPIO_R_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin5;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_GPIO_G_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin0;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_GPIO_B_Config(void)

{

GPIO_InitTypeDef  GPIO_InitStruct;

// 打开 GPIOB 端口的时钟

RCC_APB2PeriphClockCmd(LED_GPIO_CLK, ENABLE);

// 配置IO口为输出

GPIO_InitStruct.GPIO_Mode=GPIO_Mode_Out_PP;

GPIO_InitStruct.GPIO_Pin=LED_GPIO_Pin1;

GPIO_InitStruct.GPIO_Speed=GPIO_Speed_10MHz;

GPIO_Init(LED_GPIO_PORT, &GPIO_InitStruct);

}

void LED_R_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin5); //LED on

}

void LED_R_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin5); //LED off

}

void LED_G_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin0); //LED on

}

void LED_G_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin0); //LED off

}

void LED_B_ON(void)

{

// 输出电平

GPIO_ResetBits(LED_GPIO_PORT, LED_GPIO_Pin1); //LED on

}

void LED_B_OFF(void)

{

// 输出电平

GPIO_SetBits(LED_GPIO_PORT, LED_GPIO_Pin1); //LED off

}

//Bit_RESET通常为0

void LED_R_TOGGLE(void)

{

//输出翻转

if(GPIO_ReadOutputDataBit(LED_GPIO_PORT, LED_GPIO_Pin5) == Bit_RESET)//检测若为低电平输出状态（有效）则关闭灯光，若高电平（无效）则打开灯光

{

LED_R_OFF();

}

else

{

LED_R_ON();

}

}

void LED_G_TOGGLE(void)

{

//输出翻转

if(GPIO_ReadOutputDataBit(LED_GPIO_PORT, LED_GPIO_Pin0) == Bit_RESET)

{

LED_R_OFF();

}

else

{

LED_R_ON();

}

}

void LED_B_TOGGLE(void)

{

//输出翻转

if(GPIO_ReadOutputDataBit(LED_GPIO_PORT, LED_GPIO_Pin1) == Bit_RESET)

{

LED_R_OFF();

}

else

{

LED_R_ON();

}

}

Led.h

#ifndef __LED_H

#define __LED_H

#include "stm32f10x.h"

#define LED_GPIO_CLK   RCC_APB2Periph_GPIOB

#define LED_GPIO_Pin5   GPIO_Pin_5 //R

#define LED_GPIO_Pin0   GPIO_Pin_0 //G

#define LED_GPIO_Pin1   GPIO_Pin_1 //B

#define LED_GPIO_PORT   GPIOB

void LED_GPIO_R_Config(void);

void LED_GPIO_G_Config(void);

void LED_GPIO_B_Config(void);

void LED_R_ON(void);

void LED_R_OFF(void);

void LED_G_ON(void);

void LED_G_OFF(void);

void LED_B_ON(void);

void LED_B_OFF(void);

void LED_R_TOGGLE(void);

void LED_G_TOGGLE(void);

void LED_B_TOGGLE(void);

#endif