算法状态机图

VHDL代码

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Fibonacci_sequence is

Port ( n_i : in std_logic_vector(5 downto 0);

clk : in STD_LOGIC;

reset : in STD_LOGIC;

start : in STD_LOGIC;

output : out std_logic_vector(43 downto 0) );

end Fibonacci_sequence;

architecture FSMD of Fibonacci_sequence is

signal n, counter, counter_next : std_logic_vector(5 downto 0);

signal a, b, a_next, b_next : std_logic_vector(43 downto 0);

type S_Type is (ST0, ST1, ST2);

signal state, next_state : S_Type;

begin

clkproc: process(clk, reset) is

begin

if (reset= '1') then

state <= ST0;

a <= (others => '0');

b <= x"00000000001";

counter <= "000001"; -- counter从1开始计数

elsif (clk'event and clk= '1') then

state <= next_state;

a <= a_next;

b <= b_next;

counter <= counter_next;

end if;

end process clkproc;

operproc: process (state, n_i, a, b, counter, start)is

begin

output <= (others => '0');

a_next <= a;

b_next <= b;

counter_next <= counter;

case state is

when ST0 =>

if (start = '1') then

n <= n_i;

next_state <= ST1;

else

next_state <= ST0;

end if;

when ST1 =>

if (n = "000000") then

output <= (others => '0');

next_state <= ST0;

elsif (n = "000001") then

output <= x"00000000001";

next_state <= ST0;

else

a_next <= (others => '0');

b_next <= x"00000000001";

counter_next <= "000001"; -- counter从1开始计数

next_state <= ST2;

end if;

when ST2 =>

if (counter = n) then

output <= b;

next_state <= ST0;

else

a_next <= b;

b_next <= a + b;

counter_next <= counter + 1;

next_state <= ST2;

end if;

end case;

end process operproc;

end FSMD;

testbench代码

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Fibonacci_sequence_tb is

--  Port ( );

end Fibonacci_sequence_tb;

architecture tb of Fibonacci_sequence_tb is

component Fibonacci_sequence is

Port ( n_i : in std_logic_vector(5 downto 0);

clk : in STD_LOGIC;

reset : in STD_LOGIC;

start : in STD_LOGIC;

output : out std_logic_vector(43 downto 0) );

end component;

signal n : std_logic_vector(5 downto 0);

signal clk_i, res_i, start_i : STD_LOGIC := '0';

signal out_o : std_logic_vector(43 downto 0);

constant halfperiod : time := 5ns;

begin

fs: Fibonacci_sequence

port map ( n_i => n,

clk => clk_i,

reset => res_i,

start => start_i,

output => out_o

);

clk_i <= not clk_i after halfperiod;

res_i <= '1', '0' after halfperiod*4;

start_i <= '1' after halfperiod*6, '0' after halfperiod*8;

n <= "111111" after halfperiod*6;

end tb;

结果（RTL Schematic，仿真波形）

RTL Schematic:

Post-implementation simulation (Functional):

Test 1:

n = 63

可以看到，当输入的信号n为0x3f（转换到十进制为63）时，计算输出的值为0x5f6c7b064e2（转换到十进制为6557470319842）。

Test 2:

n = 9

当输入的信号n为0x09（转换到十进制为9）时，计算输出的值为0x22（转换到十进制为34）。

| number | 0 | 1 | 1 | 2 | 3 | 5 | 8 | 13 | 21 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Test 3:

当输入的信号n为0x0f（转换到十进制为15）时，计算输出的值为0x262（转换到十进制为610）。

| number | 0 | 1 | 1 | 2 | 3 | 5 | 8 | 13 | 21 | 34 | 55 | 89 | 144 | 233 | 377 | 610 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |

在上面的几次测试中，输出的值都符合斐波拉契数列的数字规律，说明电路功能正常。