代码

Combination design 1

library IEEE;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity mult5 is

port (a, b : in std_logic_vector(4 downto 0);

y: out std_logic_vector(9 downto 0));

end entity mult5;

architecture comb1_arch of mult5 is

constant WIDTH : integer := 5;

signal au, bv0, bv1, bv2, bv3, bv4: std_logic_vector(WIDTH-1 downto 0);

signal p0, p1, p2, p3, p4, prod: std_logic_vector(2*WIDTH-1 downto 0);

begin

au <= a;

bv0 <= (others => b(0));

bv1 <= (others => b(1));

bv2 <= (others => b(2));

bv3 <= (others => b(3));

bv4 <= (others => b(4));

p0 <= "00000" & (bv0 and au);

p1 <= "0000" & (bv1 and au) & '0';

p2 <= "000" & (bv2 and au) & "00";

p3 <= "00" & (bv3 and au) & "000";

p4 <= '0' & (bv4 and au) & "0000";

prod <= ((p0+p1)+(p2+p3))+p4;

y <= prod;

end architecture comb1_arch;

Combination design 1 - testbench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity adder_based_multiplier_tb is

--  Port ( );

end adder_based_multiplier_tb;

architecture tb of adder_based_multiplier_tb is

component mult5 is

port (a, b : in std_logic_vector(4 downto 0);

y: out std_logic_vector(9 downto 0));

end component;

signal a1_in, b1_in : std_logic_vector(4 downto 0);

signal y1_out : std_logic_vector(9 downto 0);

begin

ins1: mult5

port map(

a => a1_in,

b => b1_in,

y => y1_out);

a1_in <= "01001" after 10ns, "11100" after 40ns, "01010" after 70ns;

b1_in <= "10010" after 20ns, "01011" after 50ns, "10110" after 80ns;

end tb;

Combination design 2

library IEEE;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity mult5 is

port (a, b : in std_logic_vector(4 downto 0);

y: out std_logic_vector(9 downto 0));

end entity mult5;

architecture comb2_arch of mult5 is

constant WIDTH : integer := 5;

signal au, bv0, bv1, bv2, bv3, bv4: std_logic_vector (WIDTH-1 downto 0);

signal pp0, pp1, pp2, pp3, pp4: std_logic_vector (WIDTH downto 0);

signal prod: std_logic_vector (2*WIDTH-1 downto 0);

begin

au <= a;

bv0 <= (others => b(0));

bv1 <= (others => b(1));

bv2 <= (others => b(2));

bv3 <= (others => b(3));

bv4 <= (others => b(4));

pp0 <= '0' & (bv0 and au);

pp1 <= ('0' & pp0(WIDTH downto 1))+ ('0' & (bv1 and au));

pp2 <= ('0' & pp1(WIDTH downto 1))+ ('0' & (bv2 and au));

pp3 <= ('0' & pp2(WIDTH downto 1))+ ('0' & (bv3 and au));

pp4 <= ('0' & pp3(WIDTH downto 1))+ ('0' & (bv4 and au));

prod <= pp4 & pp3(0) & pp2(0) & pp1(0) & pp0(0);

y <= prod;

end architecture comb2_arch;

Combination design 2 - testbench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity adder_based_multiplier_tb is

--  Port ( );

end adder_based_multiplier_tb;

architecture tb of adder_based_multiplier_tb is

component mult5 is

port (a, b : in std_logic_vector(4 downto 0);

y: out std_logic_vector(9 downto 0));

end component;

signal a1_in, b1_in : std_logic_vector(4 downto 0);

signal y1_out : std_logic_vector(9 downto 0);

begin

ins1: mult5

port map(

a => a1_in,

b => b1_in,

y => y1_out);

a1_in <= "01001" after 10ns, "11100" after 40ns, "01010" after 70ns;

b1_in <= "10010" after 20ns, "01011" after 50ns, "10110" after 80ns;

end tb;

Repetitive-addition design

library IEEE;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity seq_mult is

port (CLK, RESET, start: in std_logic;

a_in, b_in: in std_logic_vector(7 downto 0);

ready: out std_logic;

r: out std_logic_vector(15 downto 0));

end entity seq_mult;

architecture seg_arch of seq_mult is

constant WIDTH : integer :=8;

type state_type is (idle, ab0, load, op);

signal state_reg, state_next : state_type;

signal a_reg, a_next, n_reg, n_next : std_logic_vector (WIDTH-1 downto 0);

signal r_reg, r_next : std_logic_vector (2*WIDTH-1 downto 0);

begin

-- state and data registers

process (CLK, RESET) is

begin

if RESET = '1' then

state_reg <= idle;

a_reg <= "00000000";

n_reg <="00000000";

r_reg <= x"0000";

elsif rising_edge(CLK) then

state_reg <= state_next;

a_reg <= a_next;

n_reg <= n_next;

r_reg <= r_next;

end if;

end process;

-- combinational circuit

process (start, state_reg, a_reg, n_reg, r_reg, a_in, b_in) is

begin

-- default value

a_next <= a_reg;

n_next <= n_reg;

r_next <= r_reg;

ready <= '0';

case state_reg is

when idle =>

if start = '1' then

if (a_in = "00000000" or b_in = "00000000") then

state_next <= ab0;

else

state_next <= load;

end if;

else

state_next <= idle;

end if;

ready <= '1';

when ab0 =>

a_next <= a_in;

n_next <= b_in;

r_next <= x"0000";

state_next <= idle;

when load =>

a_next <= a_in;

n_next <= b_in;

r_next <= x"0000";

state_next <= op;

when op =>

n_next <= n_reg - 1;

r_next <= ("00000000" & a_reg) + r_reg;

if (n_reg = "00000001" ) then

state_next <= idle;

else

state_next <= op;

end if;

end case;

end process;

r <= r_reg;

end architecture seg_arch;

Repetitive-addition design - testbench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity seq_mult_tb is

--  Port ( );

end seq_mult_tb;

architecture tb of seq_mult_tb is

component seq_mult is

port (CLK, RESET, start: in std_logic;

a_in, b_in: in std_logic_vector(7 downto 0);

ready: out std_logic;

r: out std_logic_vector(15 downto 0));

end component;

signal clk, reset, start, ready : std_logic := '0';

signal a_in, b_in: std_logic_vector(7 downto 0);

signal r : std_logic_vector(15 downto 0);

begin

ins: seq_mult

port map( CLK => clk,

RESET => reset,

start => start,

a_in => a_in,

b_in => b_in,

ready => ready,

r => r);

clk <= not clk after 5ns;

reset <= '0', '1' after 10ns, '0' after 20ns;

a_in <= "00000000", "00000011" after 30ns;

b_in <= "00000000", "00110111" after 30ns;

start <= '0', '1' after 40ns, '0' after 50ns;

end tb;

结果

Combination design 1 (post-implementation simulation - Timing)

Combination design 1 (RTL schematic)

Combination design 2 (post-implementation simulation - Timing)

Combination design 2 (RTL schematic)

Repetitive-addition design(post-implementation simulation-Functional)

Repetitive-addition design (RTL schematic)

分析

Combination Design 和 Repetitive-addition Design 的性能比较

1.器件利用率

Combination Design

器件利用率高：Combination design 1 和 Combination design 2 都是基于组合逻辑的乘法器设计，它们使用了多个加法器来并行计算部分积并求和。这种设计方式使得器件资源的利用率较高，因为乘法器的所有部分都在同时工作。

Repetitive-addition Design

器件利用率低：Repetitive-addition design 是基于时序逻辑的乘法器设计，它使用重复加法的方式进行乘法运算。这种设计方式在每个时钟周期内只执行一次加法操作，器件利用率相对较低。

2.延迟

Combination Design

延迟小：Combination design乘法器的运算速度较快，延迟主要由加法和部分积计算的延迟决定。例如，Combination design 1 的延迟主要来自 5 个部分积的计算和它们的求和过程，但由于是并行计算，所以总延迟相对较小。

Repetitive-addition Design

延迟大：Repetitive-addition Design乘法器的运算速度较慢，因为需要多个时钟周期来完成乘法运算。延迟由输入数的大小决定，每个时钟周期内执行一次加法操作和移位操作，因此总延迟相对较大。

3.消耗的硬件资源

- Combination Design

- 资源消耗大：由于需要同时处理所有的位运算和加法操作，组合逻辑乘法器会消耗较多的逻辑门和硬件资源。例如，在 Combination design 1 中，有 5 个部分积计算单元和一个10位的加法器来求和，导致器件资源占用较多。

Repetitive-addition Design

资源消耗小：与组合逻辑乘法器相比，重复加法乘法器的资源消耗较小。它只需要一个加法器和少量的寄存器来存储中间结果和计数器，因此在器件资源占用方面较为节省。

代码

Pipelined multiplier

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity Pipelined_multiplier is

Port (clk : in std_logic;

reset : in std_logic;

b : in std_logic_vector(8 downto 0);

a : in std_logic_vector(8 downto 0);

n : in std_logic_vector(3 downto 0);

y : out std_logic_vector(17 downto 0) );

end Pipelined_multiplier;

architecture pipe_arch of Pipelined_multiplier is

constant WIDTH : integer := 9;

signal a2_reg, a3_reg, a4_reg, a5_reg, a6_reg, a7_reg, a8_reg : std_logic_vector (WIDTH-1 downto 0);

signal b2_reg, b3_reg, b4_reg, b5_reg, b6_reg, b7_reg, b8_reg : std_logic_vector (WIDTH-1 downto 0);

signal a1, a2_next, a3_next, a4_next, a5_next, a6_next, a7_next, a8_next:std_logic_vector (WIDTH-1 downto 0);

signal b1, b2_next, b3_next, b4_next, b5_next, b6_next, b7_next, b8_next:std_logic_vector (WIDTH-1 downto 0);

signal bv0, bv1, bv2, bv3, bv4, bv5, bv6, bv7, bv8: std_logic_vector (WIDTH-1 downto 0);

signal bp0, bp1, bp2, bp3, bp4, bp5, bp6, bp7, bp8: std_logic_vector (2*WIDTH-1 downto 0);

signal pp1_reg, pp2_reg, pp3_reg, pp4_reg, pp5_reg, pp6_reg, pp7_reg, pp8_reg:std_logic_vector (2*WIDTH-1 downto 0);

signal pp0, pp1_next, pp2_next, pp3_next, pp4_next, pp5_next, pp6_next, pp7_next, pp8_next: std_logic_vector (2*WIDTH-1 downto 0);

begin

-- pipeline registers

process (clk, reset)

begin

if (reset = '1') then

pp1_reg <= (others => '0');

pp2_reg <= (others => '0');

pp3_reg <= (others => '0');

pp4_reg <= (others => '0');

pp5_reg <= (others => '0');

pp6_reg <= (others => '0');

pp7_reg <= (others => '0');

pp8_reg <= (others => '0');

a2_reg <= (others => '0');

a3_reg <= (others => '0');

a4_reg <= (others => '0');

a5_reg <= (others => '0');

a6_reg <= (others => '0');

a7_reg <= (others => '0');

a8_reg <= (others => '0');

b2_reg <= (others => '0');

b3_reg <= (others => '0');

b4_reg <= (others => '0');

b5_reg <= (others => '0');

b6_reg <= (others => '0');

b7_reg <= (others => '0');

b8_reg <= (others => '0');

elsif (clk'event and clk = '1') then

pp1_reg <= pp1_next;

pp2_reg <= pp2_next;

pp3_reg <= pp3_next;

pp4_reg <= pp4_next;

pp5_reg <= pp5_next;

pp6_reg <= pp6_next;

pp7_reg <= pp7_next;

pp8_reg <= pp8_next;

a2_reg <= a2_next;

a3_reg <= a3_next;

a4_reg <= a4_next;

a5_reg <= a5_next;

a6_reg <= a6_next;

a7_reg <= a7_next;

a8_reg <= a8_next;

b2_reg <= b2_next;

b3_reg <= b3_next;

b4_reg <= b4_next;

b5_reg <= b5_next;

b6_reg <= b6_next;

b7_reg <= b7_next;

b8_reg <= b8_next;

end if;

end process;

-- merged stage 0 & 1 for pipeline

bv0 <= (others => b(0));

bp0 <= "000000000" & (bv0 and a);

pp0 <= bp0;

a1 <= a;

b1 <= b;

--

bv1 <= (others => b1(1));

bp1 <= "00000000" & (bv1 and a1) & "0";

pp1_next <= pp0 + bp1;

a2_next <= a1;

b2_next <= b1;

-- stage 2

bv2 <= (others => b2_reg(2));

bp2 <= "0000000" & (bv2 and a2_reg) & "00";

pp2_next <= pp1_reg + bp2;

a3_next <= a2_reg;

b3_next <= b2_reg;

-- stage 3

bv3 <= (others => b3_reg(3));

bp3 <= "000000" & (bv3 and a3_reg) & "000";

pp3_next <= pp2_reg + bp3;

a4_next <= a3_reg;

b4_next <= b3_reg;

-- stage 4

bv4 <= (others => b4_reg(4));

bp4 <= "00000" & (bv4 and a4_reg) & "0000";

pp4_next <= pp3_reg + bp4;

a5_next <= a4_reg;

b5_next <= b4_reg;

-- stage 5

bv5 <= (others => b5_reg(5));

bp5 <= "0000" & (bv5 and a5_reg) & "00000";

pp5_next <= pp4_reg + bp5;

a6_next <= a5_reg;

b6_next <= b5_reg;

-- stage 6

bv6 <= (others => b6_reg(6));

bp6 <= "000" & (bv6 and a6_reg) & "000000";

pp6_next <= pp5_reg + bp6;

a7_next <= a6_reg;

b7_next <= b6_reg;

-- stage 7

bv7 <= (others => b7_reg(7));

bp7 <= "00" & (bv7 and a7_reg) & "0000000";

pp7_next <= pp6_reg + bp7;

a8_next <= a7_reg;

b8_next <= b7_reg;

-- stage 8

bv8 <= (others => b8_reg(8));

bp8 <= "0" & (bv8 and a8_reg) & "00000000";

pp8_next <= pp7_reg + bp8;

-- output

process(n, pp0, pp1_reg, pp2_reg, pp3_reg, pp4_reg, pp5_reg, pp6_reg, pp7_reg, pp8_reg)

begin

if n = "0001" then

y <= pp0;

elsif n = "0010" then

y <= pp1_reg;

elsif n = "0011" then

y <= pp2_reg;

elsif n = "0100" then

y <= pp3_reg;

elsif n = "0101" then

y <= pp4_reg;

elsif n = "0110" then

y <= pp5_reg;

elsif n = "0111" then

y <= pp6_reg;

elsif n = "1000" then

y <= pp7_reg;

elsif n = "1001" then

y <= pp8_reg;

end if;

end process;

end architecture pipe_arch;

Pipelined multiplier - testbench

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Pipelined_multiplier_tb is

--  Port ( );

end Pipelined_multiplier_tb;

architecture tb of Pipelined_multiplier_tb is

component Pipelined_multiplier is

Port (clk : in std_logic;

reset : in std_logic;

b : in std_logic_vector(8 downto 0);

a : in std_logic_vector(8 downto 0);

n : in std_logic_vector(3 downto 0);

y : out std_logic_vector(17 downto 0) );

end component;

signal clk, reset : std_logic := '0';

signal b, a : std_logic_vector(8 downto 0);

signal n : std_logic_vector(3 downto 0);

signal y : std_logic_vector(17 downto 0);

begin

ins1: Pipelined_multiplier

port map (clk => clk,

reset => reset,

b => b,

a => a,

n => n,

y => y);

clk <= not clk after 5ns;

reset <= '1', '0' after 10ns, '1' after 95ns, '0' after 100ns;

b <= "000000000", "001101001" after 10ns, "000011101" after 100ns;

a <= "000000000", "000010110" after 10ns, "000010001" after 100ns;

n <= "0000", "0111" after 10ns, "0101" after 100ns;

end tb;

结果

Pipelined design(post-implementation simulation-Functional)

Pipelined design (RTL schematic)

分析

Pipelined design的延迟：由于流水线设计将n位的乘法过程分解为多个阶段，因此总延迟由整个流水线的深度（即乘法器的位数）决定。对于n位乘法器，流水线有n个阶段，完成一次计算任务的总延迟为(n - 1)个时钟周期。

Pipelined design的吞吐量：对于n位乘法器，完成一次计算任务的总延迟为(n - 1)个时钟周期。由于流水线设计的特性，完成k个n位计算任务需要的总时长为(n + k - 2)个时钟周期T，吞吐量为k / ((n + k - 2) * T)，当k很大时吞吐量近似于(1 / T)。

和Combination design进行比较：

硬件资源：Combination Design在硬件资源方面具有较高的逻辑门和加法器资源消耗，但寄存器使用较少，适合对延迟要求极高的应用场景。

Pipelined Design在硬件资源方面具有适量的加法器和大量的寄存器资源，适合需要高吞吐量的应用场景。虽然总延迟较大，但每个阶段的延迟较小，且能够通过流水线的方式提高运算效率。

和Repetitive-addition Design进行比较：

吞吐量比较：Repetitive-addition design的吞吐量较低，因为它在一个乘法运算完成之前无法开始新的运算。而Pipelined design能够在一个时钟周期内启动一个新的乘法运算，从而具有更高的吞吐量。