期末测试

姓名：何衍宁 ；

时间：2025/12/24;

- 用Atlas仿真器仿真出一个硅基半导体电阻，要求：

（1）尺寸自行定义；

（2）阻值约为2K欧姆；

（3）给出伏安特性关系图（电压范围0~4V）；

1 硅基半导体电阻仿真设计

我们通过代码设定的参数如下：x轴和y轴的范围分别为10.00um和100.00um，而z轴的深度则采用软件的默认值。在此基础上，硅材料的掺杂浓度被设定为1.45612×10¹⁸。同时，为了更准确地模拟器件的物理特性，我们选择了Conmob（浓度依赖迁移率）和Fldmob（平行电场依赖迁移率）模型作为仿真过程中的关键模型。这些参数和模型的组合，共同构建了我们所研究的目标器件——硅基半导体电阻。

2 伏安特性关系图（电压范围0~4V）

第一张图片是通过仿真得出的伏安特性关系图，第二张图片是通过将仿真得出的数据导入matlab后经过线性拟合划出的图像，我们可以得出电阻的数值为斜率分之一，约为2k欧姆，符合题目要求

3 代码展示

go atlas

mesh space.mult=1.0

x.mesh location=0.00 spac=1.0

x.mesh location=10.00 spac=1.0

y.mesh location=0.00 spac=1

y.mesh location=100.00 spac=1

region num=1 silicon

electrode name=anode top

electrode name=cathode bottom

doping n.type conc=1.45612e18 uniform

model conmob fldmob

solve init

method newton

structure outfile=resistor.str

log outfile=resistor.log

solve vanode=0 vstep=0.05 vfinal=4 name=anode

extract name="I_at_0V" y.val from curve(abs(v."anode"),abs(i."anode")) where x.val=0.0

extract name="I_at_1V" y.val from curve(abs(v."anode"),abs(i."anode")) where x.val=1.0

extract name="I_at_2V" y.val from curve(abs(v."anode"),abs(i."anode")) where x.val=2.0

extract name="I_at_3V" y.val from curve(abs(v."anode"),abs(i."anode")) where x.val=3.0

extract name="I_at_4V" y.val from curve(abs(v."anode"),abs(i."anode")) where x.val=4.0

tonyplot resistor.str

tonyplot resistor.log

quit