问题a：

fc = 77e9;

B =150*10^6;

Tchirp = 8*10^(-6);

K = B / Tchirp;

c = 3e8;

Nr = 1024;

Nd = 128;

range_res = 1;

signal = reshape(x1,Nr,Nd);

% 距离维FFT

sig_fft = fft(x1(1:1024))./Nr;

sig_fft = abs(sig_fft);

sig_fft = sig_fft(1:(Nr/2));

figure;

plot(sig_fft);

xlabel('距离（频率）');

ylabel('幅度')

title('第一个chirp的FTF结果')

%距离：89m

%精度：1m和1.9024m/s

问题b：

fc = 77*10^9;

B =150*10^6;

Tchirp = 8*10^(-6);

K = B / Tchirp;

c = 3e8;

lamuda = c/fc;

range_res = 1;

Nr = 1024;

Nd = 128;

vel_res = lamuda/(2*Tchirp*Nd);

Mix_fft1d = reshape(x2, [Nr, Nd]);

%%原始信号

figure(1);

subplot(2,1,1);

plot(abs((Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

subplot(2,1,2);

plot(abs(fft(Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

% 两个fft搭建fft2

% signal_fft = fft(Mix_fft1d, Nr);

% figure(2)

% mesh(db(abs(signal_fft./max(signal_fft))));

% title('Range from First FFT');

% ylabel('Range [m]');

% xlabel('chirps[N]')

% zlabel('Amplitude (dB)');

% title('距离维FFT')

% for i=1:Nr

%     signal_fft2(i,:) = fftshift(fft(signal_fft(i,:)));

% end

signal_fft2 = fft2(Mix_fft1d,1024,128);

for i = 1:Nr/2 % 速度包含正负，所以对速度fftshift；距离直接截一半

signal_fft3(i,:) = fftshift(signal_fft2(i,:));

end

doppler_axis = vel_res*((-Nd/2:Nd/2-1));

range_axis = range_res*((1:Nr/2));

figure(3);

mesh(doppler_axis,range_axis,db(abs(signal_fft3)));

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

title('Amplitude and Range From FFT2');

% CFAR implementation

RDM =(abs(signal_fft3));

n_train_cells = 8;

n_train_bands = 8;

%test (CUT) for accurate estimation

n_guard_cells = 4;

n_guard_bands = 4;

offset = 1.4;

noise_level = zeros(1,1);

RDM = RDM / max(RDM(:));

RDM(RDM~=0 & RDM~=1) = 0;

figure('Name', 'CA-CFAR Filtered RDM')

mesh(doppler_axis,range_axis,RDM);

title( 'CA-CFAR Filtered RDM surface plot');

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

%%$ ending % 11.4144m/s;89m

% 1-512m

% -1.9024*64~1.9024*63 = -121.7532~119.8509

问题c：

无

问题d：

clear all;

clc;

close all;

%% Radar Specifications

fc = 77*10^9;

B =150*10^6;

Tchirp = 8*10^(-6);

K = B / Tchirp;

c = 3e8;

lamuda = c/fc;

range_res = 1;

Nr = 1024;

Nd = 128;

vel_res = lamuda/(2*Tchirp*Nd);

%% User Defined Range and Velocity of target

range = 14;

vel = 10;

range1 = 10;

vel1 = 14;

max_vel = lamuda/(4*Tchirp);% m/s

%% FMCW Waveform Generation

B = c / (2*range_res);

slope = B/Tchirp;

t=linspace(0,Nd*Tchirp,Nr*Nd); %total time for samples

r_t=zeros(1,length(t));

td = 2 * (range + vel*t)/ c;

td1 = 2 * (range1 + vel1*t)/ c;

Tx  = cos(2*pi*(fc*t + (slope*t.*t)/2 ) );

Rx  = cos(2*pi*(fc*(t -td) + (slope * (t-td).*(t-td))/2 ) );

Rx1  = cos(2*pi*(fc*(t -td1) + (slope * (t-td1).*(t-td1))/2 ) );

% Mix = Tx.*Rx;%混频

% Mix1 = Tx.*Rx1;

% Mix = Mix + Mix1;

Mix = Tx.*(Rx+Rx1);

% Mix_fft1d =Mix+awgn(Mix,20);%增加高斯白噪声

Mix_fft1d = reshape(Mix, [Nr, Nd]);

%%原始信号

figure(1);

subplot(2,1,1);

plot(abs((Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

subplot(2,1,2);

plot(abs(fft(Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

[pks1,locs1] = findpeaks(abs(fft(Mix_fft1d(:,1))));

cnt = 1;

for i = 1:length(locs1)

if(pks1(i)>max(pks1)*0.9 && locs1(i) <= 512)

locs2(cnt) = locs1(i);

cnt = cnt + 1;

end

end

signal_fft = fft(Mix_fft1d, Nr);

figure(2)

mesh(db(abs(signal_fft./max(signal_fft))));

title('Range from First FFT');

ylabel('Range [m]');

xlabel('chirps[N]')

zlabel('Amplitude (dB)');

title('距离维FFT')

signal_fft2 = zeros(512,128);

for i=1:Nr/2

signal_fft2(i,:) = fftshift(fft(signal_fft(i,:)));

end

% for i=1:length(locs2)

%     signal_fft2(locs2(i),:) = fftshift(fft(signal_fft(locs2(i),:)));

% end

% signal_fft2 = fft2(Mix_fft1d,1024,128);

% for i = 1:Nr/2 % 速度包含正负，所以对速度fftshift；距离直接截一半

%     signal_fft3(i,:) = fftshift(signal_fft2(i,:));

% end

doppler_axis = vel_res*((-Nd/2:(Nd-1)/2)-1);

range_axis = range_res*((1:Nr/2)-1);

figure(3);

mesh(doppler_axis,range_axis,db(abs(signal_fft2)));

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

title('Amplitude and Range From FFT2');

%% CFAR implementation

RDM =db(abs(signal_fft2));

n_train_cells = 8;

n_train_bands = 8;

%test (CUT) for accurate estimation

n_guard_cells = 4;

n_guard_bands = 4;

offset = 1.4;

noise_level = zeros(1,1);

for i = 1:length(locs2)

[~,v_r(i)] = max(RDM(locs2(i),:));

RDM(locs2(i),:) = 0;

RDM(locs2(i),v_r(i)) = 1;

end

for i = 1 : Nr/2

if(i ~= locs2(1) && i ~= locs2(2))

RDM(i,:)=0;

end

end

v_r = (v_r - 65 - 1) * vel_res;

r_r = locs2 - 1;

% RDM(RDM~=0 & RDM~=1) = 0;

% RDM(RDM<0.98) = 0;

% count = 0;

% for i=1:512

%     for j=1:128

%         if(RDM(i,j)~=0)

%             count = count + 1;

%         end

%     end

% end

figure('Name', 'CA-CFAR Filtered RDM')

mesh(doppler_axis,range_axis,RDM);

title( 'CA-CFAR Filtered RDM surface plot');

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

%%$ ending

问题e：

clear all;

clc;

close all;

%% Radar Specifications

fc = 77*10^9;

B =150*10^6;

Tchirp = 8*10^(-6);

K = B / Tchirp;

c = 3e8;

lamuda = c/fc;

range_res = 1;

Nr = 1024;

Nd = 128;

vel_res = lamuda/(2*Tchirp*Nd);

leidad = 10;

%% User Defined Range and Velocity of target    雷达1

range11 = 14;

vel11 = 10;

range21 = 10;

vel21 = 14;

max_vel = lamuda/(4*Tchirp);% m/s

% FMCW Waveform Generation

B = c / (2*range_res);

slope = B/Tchirp;

t=linspace(0,Nd*Tchirp,Nr*Nd); %total time for samples

r_t=zeros(1,length(t));

td = 2 * (range11 + vel11*t)/ c;

td1 = 2 * (range21 + vel21*t)/ c;

Tx  = cos(2*pi*(fc*t + (slope*t.*t)/2 ) );

Rx  = cos(2*pi*(fc*(t -td) + (slope * (t-td).*(t-td))/2 ) );

Rx1  = cos(2*pi*(fc*(t -td1) + (slope * (t-td1).*(t-td1))/2 ) );

Mix = Tx.*Rx;%混频

Mix1 = Tx.*Rx1;

Mix = Mix + Mix1;

% Mix_fft1d =Mix+awgn(Mix,20);%增加高斯白噪声

Mix_fft1d = reshape(Mix, [Nr, Nd]);

%%原始信号

figure(1);

subplot(2,1,1);

plot(abs((Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

subplot(2,1,2);

plot(abs(fft(Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

[pks1,locs1] = findpeaks(abs(fft(Mix_fft1d(:,1))));

cnt = 1;

for i = 1:length(locs1)

if(pks1(i)>max(pks1)*0.9 && locs1(i) <= 512)

locs2(cnt) = locs1(i);

cnt = cnt + 1;

end

end

signal_fft = fft(Mix_fft1d, Nr);

figure(2)

mesh(db(abs(signal_fft./max(signal_fft))));

title('Range from First FFT');

ylabel('Range [m]');

xlabel('chirps[N]')

zlabel('Amplitude (dB)');

title('距离维FFT')

signal_fft2 = zeros(512,128);

for i=1:length(locs2)

signal_fft2(locs2(i),:) = fftshift(fft(signal_fft(locs2(i),:)));

end

% signal_fft2 = fft2(Mix_fft1d,1024,128);

% for i = 1:Nr/2 % 速度包含正负，所以对速度fftshift；距离直接截一半

%     signal_fft3(i,:) = fftshift(signal_fft2(i,:));

% end

doppler_axis = vel_res*((-Nd/2:(Nd-1)/2)-1);

range_axis = range_res*((1:Nr/2)-1);

figure(3);

mesh(doppler_axis,range_axis,db(abs(signal_fft2)));

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

title('Amplitude and Range From FFT2');

% CFAR implementation

RDM =db(abs(signal_fft2));

n_train_cells = 8;

n_train_bands = 8;

%test (CUT) for accurate estimation

n_guard_cells = 4;

n_guard_bands = 4;

offset = 1.4;

noise_level = zeros(1,1);

for i = 1:length(locs2)

[~,v_r1(i)] = max(RDM(locs2(i),:));

RDM(locs2(i),:) = 0;

RDM(locs2(i),v_r1(i)) = 1;

end

v_r1 = (v_r1 - 65 - 1) * vel_res;

r_r1 = locs2 - 1;

%% User Defined Range and Velocity of target    雷达2

range12 = 22;

vel12 = 10;

range22 = 14;

vel22 = 14;

max_vel = lamuda/(4*Tchirp);% m/s

% FMCW Waveform Generation

B = c / (2*range_res);

slope = B/Tchirp;

t=linspace(0,Nd*Tchirp,Nr*Nd); %total time for samples

r_t=zeros(1,length(t));

td = 2 * (range12 + vel12*t)/ c;

td1 = 2 * (range22 + vel22*t)/ c;

Tx  = cos(2*pi*(fc*t + (slope*t.*t)/2 ) );

Rx  = cos(2*pi*(fc*(t -td) + (slope * (t-td).*(t-td))/2 ) );

Rx1  = cos(2*pi*(fc*(t -td1) + (slope * (t-td1).*(t-td1))/2 ) );

Mix = Tx.*Rx;%混频

Mix1 = Tx.*Rx1;

Mix = Mix + Mix1;

% Mix_fft1d =Mix+awgn(Mix,20);%增加高斯白噪声

Mix_fft1d = reshape(Mix, [Nr, Nd]);

%%原始信号

figure(1);

subplot(2,1,1);

plot(abs((Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

subplot(2,1,2);

plot(abs(fft(Mix_fft1d(:,1))));

xlabel('采样点数(N)')

ylabel('幅度(A)')

[pks1,locs1] = findpeaks(abs(fft(Mix_fft1d(:,1))));

cnt = 1;

for i = 1:length(locs1)

if(pks1(i)>max(pks1)*0.9 && locs1(i) <= 512)

locs2(cnt) = locs1(i);

cnt = cnt + 1;

end

end

signal_fft = fft(Mix_fft1d, Nr);

figure(2)

mesh(db(abs(signal_fft./max(signal_fft))));

title('Range from First FFT');

ylabel('Range [m]');

xlabel('chirps[N]')

zlabel('Amplitude (dB)');

title('距离维FFT')

signal_fft2 = zeros(512,128);

for i=1:length(locs2)

signal_fft2(locs2(i),:) = fftshift(fft(signal_fft(locs2(i),:)));

end

% signal_fft2 = fft2(Mix_fft1d,1024,128);

% for i = 1:Nr/2 % 速度包含正负，所以对速度fftshift；距离直接截一半

%     signal_fft3(i,:) = fftshift(signal_fft2(i,:));

% end

doppler_axis = vel_res*((-Nd/2:(Nd-1)/2)-1);

range_axis = range_res*((1:Nr/2)-1);

figure(3);

mesh(doppler_axis,range_axis,db(abs(signal_fft2)));

xlabel('Speed[m/s]');

ylabel('Range[m]');

zlabel('Amplitude[dB]');

title('Amplitude and Range From FFT2');

% CFAR implementation

RDM =db(abs(signal_fft2));

n_train_cells = 8;

n_train_bands = 8;

%test (CUT) for accurate estimation

n_guard_cells = 4;

n_guard_bands = 4;

offset = 1.4;

noise_level = zeros(1,1);

for i = 1:length(locs2)

[~,v_r2(i)] = max(RDM(locs2(i),:));

RDM(locs2(i),:) = 0;

RDM(locs2(i),v_r2(i)) = 1;

end

v_r2 = (v_r2 - 65 - 1) * vel_res;

r_r2 = locs2 - 1;

%%

num = length(v_r1);

leida1 = sortrows([v_r1;r_r1]');

leida2 = sortrows([v_r2;r_r2]');

leida11 = leida1(:,2)';

leida22 = leida2(:,2)';

for i=1:num % 以雷达1为基准

angle(i) = acos((leida11(i)^2+leidad^2-leida22(i)^2)/(2*leida11(i)*leidad));

end

leida = leida11;

disp(leida);

disp(angle);